POV 3D HOLOGRAM DISPLAY SYSTEM (v4)

Complete Engineering Documentation, Circuit Schematics, Timing Physics & Poster Specifications
Designed & Developed by: Aryan Khona

# 1. Hardware & Pin Configuration

| Component | Pin on ESP8266 | Details & Function |
| --- | --- | --- |
| WS2812 Strip-1 | D1 (GPIO 5) | 8 LEDs (Blade A, high-speed 800kHz bitbang data line) |
| WS2812 Strip-2 | D2 (GPIO 4) | 8 LEDs (Blade B, opposite rotor blade 180° offset) |
| Hall Effect Sensor | D7 (GPIO 13) | 1 pulse / revolution (A3144 interrupt zero-index trigger) |
| Battery Monitoring | A0 (ADC0) | 10kΩ/10kΩ 1:1 voltage divider, calibrated for 3.6V to 4.2V |
| High-Speed Motor | Separate Supply | Dedicated 3.5V supply for stable rotor rotation (900-1300 RPM) |

# 2. Circuit Architecture & Power Distribution

The electrical circuit consists of two isolated, independently powered subsystems:
1. Stator Motor Drive: A dedicated 4.2V Li-ion battery feeds a TP4056 USB charging module, controlled by an ON/OFF switch. A 1N4007 protection diode steps down the output to 3.5V for the DC motor. A neodymium magnet is fixed to the stator frame.
2. Rotor Display Subsystem: A separate on-rotor 4.2V Li-ion battery and TP4056 module supply power through an ON/OFF switch and 1N4007 diode down to 3.5V (+Vcc), powering both the NodeMCU ESP8266 and the two WS2812 LED strips. The A3144 Hall sensor connects to +Vcc, GND, and D7. Pin A0 taps the midpoint of a 10kΩ/10kΩ resistor divider across +Vcc and GND.

# 3. Wi-Fi & Network Architecture

| Mode | SSID | Password | Default IP Address | Operational Role |
| --- | --- | --- | --- | --- |
| Station Mode (STA) | POV_Hologram_STA_MODE | 013171317 | 10.218.184.220 | Connects to existing home/lab Wi-Fi network |
| Access Point (AP) | POV_Hologram_AP_MODE | 013171317 | 192.168.4.1 | Autonomous field hotspot if STA fails |

• Web Server Port: 80
• STA Connection Timeout: 15 seconds
• Automatic Failover: If the STA router connection cannot be established within 15 seconds, Access Point (AP) mode launches automatically.

# 4. Operational Display Modes

| Mode ID | Mode Name | Primary Function & Capabilities |
| --- | --- | --- |
| 0 | Text | Custom static & scrolling text input (dynamic letter limit, 5x5 font, anti-ghosting blanking) |
| 1 | Canvas | User drawing / custom graphics (100 columns, flat & polar modes, EEPROM storage) |
| 2 | Digital Clock | 12/24 hour digital time readout with live phone epoch synchronization |
| 3 | Analog Clock | Virtual clock face with hours, minutes, second sweep hand, and dial tick markers |
| 4 | Date & Day | Smooth automated alternation between 3-letter weekday abbreviation and date/month |
| 5 | Shapes | Parametric vector icons: Heart (scalable width/height) and smiley face with color pickers |
| 6 | Auto Mode | Autonomous carousel loop cycling through all enabled modes at 2 to 60 second intervals |
| 7 | Sentence (Ticker) | New in v4: Long sentences (up to 100 chars), angular aperture window (0°-270°), dynamic rainbow cycling |

# 5. RPM, LED Synchronization & Timing Parameters

| Parameter | Value / Mathematical Calculation | Technical Purpose |
| --- | --- | --- |
| Good Display RPM | >= 900 RPM | Provides stable, flicker-free persistence of vision |
| Revolution Time (T_rev) | T_rev = 60 / RPM | Total duration for one complete 360° rotor turn |
| Example Benchmark RPM | 1200 RPM | Ideal rotational benchmark speed |
| T_rev at 1200 RPM | T_rev = 60 / 1200 = 0.05 s = 50 ms | 50,000 microseconds per rotation |
| Canvas Columns | 100 columns | Cylindrical space partitioned into 100 angular slices |
| Angle per Column | Angle = 360° / 100 = 3.6° | Angular width of each column strobe |
| LED Column Time | T_column = 50 ms / 100 = 0.5 ms (500 µs) | Approximate time budget per column at 1200 RPM |
| LED Latch Time | 60 µs to 80 µs | Reset signal duration to latch WS2812 pixel data |
| Hall Pulse Filter | 2 ms | Debounce window to reject magnetic jitter and noise |
| Motor Stop Detection | 1 second | Safely blanks LEDs when rotor is stationary |
| CPU Core Frequency | 160 MHz | Eliminates timing jitter during Wi-Fi and FastLED operations |

# 6. Language & Technology Stack

| Technology / Tool | Application in Project |
| --- | --- |
| C++ (Arduino C++) | Core ESP8266 firmware, fast dual-blade bitbang driver, predictive timing filter |
| HTML / CSS | Responsive web dashboard structure and modern dark UI theme |
| JavaScript | Interactive dashboard controls, real-time telemetry, and canvas rendering engine |
| Arduino IDE | Compilation environment, ESP8266 core, and flash toolchain |
| ESP8266 Web Server | Lightweight HTTP server running on port 80 |
| NTP | Network Time Protocol for automatic atomic clock synchronization |
| EEPROM Emulation | Persistent non-volatile storage for custom text, modes, and drawings |
| OTA (Over-The-Air) | Wireless firmware uploading directly over Wi-Fi without USB cables |

# 7. 17 Core Engineering Features

• 1. Wi-Fi web dashboard

• 2. Custom text/sentence input

• 3. Automatic looping text scrolling

• 4. Adjustable scrolling speed

• 5. Adjustable hold time & letter gap

• 6. Brightness and colour control

• 7. Horizontal / vertical flip

• 8. Display angle adjustment

• 9. Digital & analog clock

• 10. Date & day display

• 11. Heart & smiley shapes

• 12. Automatic mode cycling

• 13. Real-time RPM monitoring

• 14. Battery percentage monitoring

• 15. Motor jitter / quality monitoring

• 16. EEPROM setting storage

• 17. OTA (Over-The-Air) firmware update

Report generated for GitHub Pages & Project Showcase by Aryan Khona.